1. Audio capture (AVB or analog tap)

If your conference microphones already feed an amplifier or DSP (for example a Biamp or Q-Sys unit), you can mirror that feed to the PC. There are two options:

A. Analog tap

- Use a balanced line-out or aux send from the DSP into a USB audio interface.

- Interfaces like the Focusrite Scarlett Solo (3rd Gen) cost around $130 and provide clean stereo input directly to Teams or OBS.

- Cabling: balanced ¼-inch TRS or XLR cables and a ground-isolating transformer if needed ($25-$50).

B. AVB or Dante tap

- If the audio backbone is networked (Biamp Tesira AVB, Dante-enabled mixer, etc.), you can capture it with a Dante Virtual Soundcard or MOTU AVB USB interface.

- Software license (Dante VSC) ≈ $60 per machine.

- Hardware interface (MOTU Ultralite AVB) ≈ $595 if you need a physical bridge.

- This yields pristine digital audio synchronized with the meeting.

Practical pick: if the DSP already has an analog output, use the analog tap; it’s cheaper and simpler.

2. Camera and video input

Because Teams and most streaming apps accept USB video:

- Basic fixed camera: Logitech Brio 4K ≈ $130 (wide angle, plug-and-play).

- PTZ camera for room coverage: PTZOptics 20x USB ≈ $1,499 or Aver VC520 ≈ $800.

- Mounting hardware + cables: $100-$200.

If you want higher production control (switching, overlaying slides, or adding captions), pair the camera with OBS Studio (free) running on the existing PC.

3. Cloud storage and archiving

Assuming one meeting per week, two hours each, recorded at 1080p (≈ 1.5 GB per hour):

- Annual data volume ≈ 150 GB.

- Microsoft OneDrive Business (1 TB) = $60 per user per year.

- Google Workspace Business Standard (2 TB) = $144 per year.

- AWS S3 Standard ≈ $0.023/GB/month → ~ $41/year for 150 GB (+ egress).

- Optional redundancy: replicate recordings to YouTube (unlisted) at no cost.

Add a simple folder policy: retain six months of full video, one year of audio-only, unless retention law requires longer.

4. Streaming method

If you only need public viewing (not participation):

- YouTube Live: free, automatically archives streams.

- Facebook Live: free, limited analytics.

- Teams Live Event: already included in Microsoft 365 E3/E5 licenses.

- OBS → YouTube or Facebook: free and simple.

Each option works with the existing PC and internet line (10 Mbps upload is sufficient).

5. Estimated total cost

| Item | One-Time | Annual |
| --- | --- | --- |
| Audio interface + cables | $150 | – |
| Camera (HD/PTZ) | $130 – $1,500 | – |
| Mounting + misc hardware | $150 | – |
| Cloud storage (1 TB) | – | $60 – $150 |
| Streaming software/service | – | Free – $100 |
| Totals | $300 – $1,800 initial | $60 – $250 annual |

Implementation summary

Jerome County could launch permanent live-streaming and archiving of commission meetings for under $2,000 startup and under $250 per year thereafter, using the hardware already in the room. The simplest configuration:

- Analog tap from existing microphones into USB audio interface.

- HD webcam or PTZ camera via USB.

- OBS Studio or Teams Live Event for streaming/recording.

- Cloud storage or YouTube archive for public access and retention.